## Introduction

We use two methods to estimate percent replacement of metamict zircon by crystalline zircon. Backscattered electron or cathodoluminescence greyscale images collected on the SEM with high contrast between metamict and crystalline zones can be thresholded in ImageJ to produce a binary image. The number of pixels in the metamict zone can be counted and divided by the total number of pixels to obtain area%. However, this approach is somewhat arbitrary, as the % replacement depends on the threshold level. A bigger problem is that the 2D section is unlikely to be representative of the bulk sample, so the area % obtained will not equal the volume %. This problem is overcome through use of the isotope dilution methods, which measures the shift in isotopic composition of the fluid as a function of the total volume of metamict zircon replaced.

## Methods

### Experiments

Zircon solubility measurements were made at 0.2 GPa and 450, 600, and 700 °C. Mud Tank zircon was cut into 1-2 mm diameter cubes using a DISCO saw (DAD3220 dicing saw, Disco, Japan). Cubes were cleaned, optically inspected for impurities, dried and weighed. Two cubes were placed in a 3mm o.d. Pt inner capsule ~1.2 cm in length. We used The Geochemists Workbench React program and the 0.2 GPa thermodynamic database of Ayers and Zhu (2025) to calculate the amount of SiO2 needed to saturate the solution in the outer capsule in quartz and added that amount of powdered 29SiO2 or 30SiO2. The weighed inner capsule was placed in a weighed 5 mm o.d. ~3 cm long Pt capsule tri-crimped and welded on one end. A microsyringe was used to add 120 μL of USGS49 Antarctic Ice core water (δ2H = -394.7‰, δ18O -50.55‰) ± 1m NaF to the outer capsule, which was flat-crimped and welded shut using a Lampert PUK-06 microwelder. The capsule was weighed, placed in a 110 °C oven overnight, and weighed again to test the weld integrity.

Capsules were run in cold seal pressure vessel systems. Samples were quenched by raising the furnace and using a fan to blow air on the pressure vessel (<5 min to reach ambient temperature). The capsule was retrieved, weighed, and then punctured. A glass microsyringe was used to extract solution, which was immediately injected into a vial. The vial was topped off with East Nashville Tap water with δ2H = -40.6‰, δ18O -6.59‰ to eliminate headspace and bring the isotopic composition within the calibrated range and then weighed.

The inner capsule was removed, dried, and weighed before opening and collecting the zircon cubes. The cubes were cleaned, dried, and weighed. One cube was placed face-down on double-stick tape and covered with epoxy. The 1” o.d. cured epoxy mount was ground into to expose a cross-section, then polished to 1 μm and carbon-coated. Quenched solutes were collected from the outer capsule, dried, weighed, and then mounted in epoxy and polished for SIMS analysis.

### Chemical and isotopic analysis

Before the experiments, backscattered electron and cathodoluminescence images of zircon cubes were collected on a Tescan Vega 3 LM environmental scanning electron microscope with 10kV accelerating voltage. An Oxford x-Max energy dispersive spectrometer was used to collect chemical analysis of the starting materials and run products. A dwell time of 15 seconds was used for point analyses and linescans. The beam was optimized internally using Aztec version 4.1 on pure copper tape, and USGS RGM-1 glass was run daily as a standard.

#### Imaging and major element characterization in run products using FIB-SEM

After the experiments zircon cubes were mounted in 1” epoxy round mounts. Prior to analysis, each sample was polished and then sputter coated (EMS Quorum/Q 150T ES plus, Quorum, UK) with 10 nm graphite powder. Morphological characteristics and imaging of the prepared sample mounts were obtained by using a focused ion beam scanning electron microscope (FEI Helios NanoLab G3 CX, FEI Company, USA) equipped with a concentric backscatter detector in the core facilities of the Vanderbilt Institute of Nanoscale Sciences and Engineering (VINSE).

The analyses of the major elements were performed using an X-max 50^2 SDD energy-dispersive x-ray spectrometer (EDS) manufactured by Oxford Instrument, UK. Point analysis was performed on the interested spots of the sample to determine the concentrations of Na, Mg, Al, Si, P, K, Ca, Ti, Cr, Mn, Fe, Zr and Ni. The oxygen concentration was determined by stoichiometric calculation assuming all the elements were oxides. To determine the signal of trace elements like Th (Lα: 12.97 eV), and U (Lα: 13.61 eV), it is necessary to overshoot the critical ionization energy by a margin, usually 1.5–3 times the desired X-ray energy. Secondary and back-scatter electron imaging were carried out at 15 keV and 0.34 nA with a working distance of 6.0 mm, while the EDS analysis was carried out at 15~20 keV and 0.34 nA at 6.0 mm working distance. The beam was optimized internally using Clif-Lorimer standardless method in Aztec (3.1).

Micro-Raman measurements were conducted with a Thermo Scientific DXR Confocal Raman microscope in the core facilities of the Vanderbilt Institute of Nanoscale Sciences and Engineering (VINSE). A microscope is equipped with a 532 nm (10 mW) and 780 nm (24 mW) depolarized lasers with a power regulated in 0.1 mW. It has a set of 10X, 50X and 100X objectives and a 25/50 µm spectrograph apertures. The locations of the scanning spots have been captured with a digital camera. The lateral spot size could be controlled by objectives and can be as small as 0.6 µm for 100x objective. The tool calibration is done by on-board neon and polystyrene standards. All measurements were made using a 50 um aperture slit.

Oxygen isotope analysis of 29SiO2 and 30SiO2 purchased from Isoflex was performed using Laser Fluorination by Ilya Bindeman at University of Oregon. Measured values normalized to SMOW were δ18O = -19.25‰ for 29SiO2 and +20.77‰ for 30SiO2.

Zircon samples were analyzed for Si and O isotopes using a Cameca ims 7f at Caltech. Spot size was 20-25 μm diameter. Oxygen isotope analyses were performed first using a cesium source, and then Si isotope analyses were collected using a duoplasmotron source. Measured isotopic ratios were corrected for instrumental mass fractionation by applying a correction factor calculated from the analyses of Mudtank zircon reference material with known 18O/16O ratio and 29Si/28Si ratio. The measured values of δ18O for the zircon standard Mudtank is 5.03 ± 0.20 ‰ (referenced to SMOW) and δ29Si for Mudtank is -0.19 ± 0.11 ‰ (referenced to NBS28). Measurements of isotopic ratio were background-corrected. The average value of the correction factor determined by multiple measurements of standard Mud Tank zircon during the same analytical session was used as the primary standard to correct the background-corrected measured isotopic ratios of the samples. Use oxygen isotope as an example, (18O/16O)t,s = (18O/16O)m,s * CF, where CF = (18O/16O)t,std/(18O/16O)m,std and t = true, m = measured, s = sample, and std = standard. Periodic measurement of isotopic ratios of Mudtank of homogenous zircon domains over the course of an analytical session showed no significant drift over time. The corrected 18O/16O and 29Si/28Si ratios of samples were then expressed in delta notation.

The δ18O isotope values of run product solutions were determined using a Picarro L2130-i isotope and gas concentration analyzer at Vanderbilt University. Each sample was analyzed twice, with four measured injections (following four preparatory injections) for each run. The average values of the two sample runs are reported in delta notation relative to Vienna Standard Mean Ocean Water (VSMOW). The data sets were corrected for machine drift during the run and normalized to the USGS 50/USGS 49 scale by assigning values of 4.95‰ and -50.55‰ (δ18O) to USGS 50 and USGS 49 through the USGS LIMS for Lasers data reduction scheme (Coplen, 1998). The external analytical reproducibility for the laser spectrometer, defined as the standard deviation of control standard (USGS 44 and in-house standard ENT-2) during all runs, yielding an average precision (1σ) of ~0.02‰ for δ18O between replicate sample runs.

### Solubility estimated using isotope dilution

In the experiment the 30SiO2 powder added to the outer capsule dissolves rapidly to make a quartz-saturated solution. The Mud Tank zircon will dissolve much more slowly than the silica powder, and as it does the Si it adds to the solution causes quartz oversaturation and precipitation. The zircon will continue to dissolve, and δ30Sisoln and δ18Osoln will decline, until the solution becomes saturated in zircon. The isotopic composition of the fluid is unlikely to change once zircon saturation is reached because isotopic exchange will be limited by the very low diffusivities of Si and O in zircon.

To estimate the solubility of zircon, we start with an equation for O18/O16 for the final solution soln. When the experiment ends, there are contributions of oxygen in the solution from four phases: dissolved zircon z, USGS49 water w, SiO2, and East Nashville tap water ENT for dilution for Picarro analysis. If we measure the oxygen isotope composition of the quenched solutes using SIMS, we could substitute those values for the solution values, since the isotopic composition of the quenched solutes should equal that of the final solution.

Rsoln=O18O16soln= Oz18+Ow18+OSiO218+OENT18Oz16+Ow16+OSiO216+OENT16	(1)

where O18z is the number of atoms of O18 contributed to QS by the zircon dissolution, which is equal to:

Oz18=Xz18CzOmz	(2)

where X18z is the atomic fraction of O18 in z, COz is the atomic concentration of oxygen in z (ZrSiO4 = 4/6 = 0.66), and mz is the number of molecules of zircon dissolved in the solution at equilibrium.

mz=CwOmwXw18-RsolnXw16+CSiO2OmSiO2XSiO218-RsolnXSiO216+CENTOmENTXENT18-RsolnXENT16CzO(RsolnXz16-Xz18)	(6)

We can write a similar equation for Si isotopes but without the w and ENT terms. This approach requires that all SiO2 added to the capsule dissolves in the solution, although the error from a small amount of quartz being present during the experiment will be small because Rqz will be relatively close to Rsoln if they were in isotopic equilibrium.

### Mass of metamict ZrSiO4 replaced

Replacement can be quantified because metamict zircon mz is replaced with crystalline zircon z with a very different isotopic composition. In the experiment the 30SiO2 added to the outer capsule dissolves rapidly to make a solution slightly undersaturated in quartz. The metamict zircon will start dissolving, adding Zr and Si to the solution. If any quartz precipitates, it will have roughly the same isotope composition as the solution. As metamict zircon continues to dissolve, the solution becomes oversaturated in crystalline zircon, and the concentrations of Zr and Si in the solution will be maintained at a steady state during coupled dissolution-precipitation. The Si isotope composition of the solution δ30Sisoln will decrease continuously. At the end of the experiment, the Si isotope composition of the solution and quenched solutes will be the same, and are determined by the amounts and isotopic abundances of metamict ZrSiO4 dissolved mz, 30SiO2 added, and crystalline zircon precipitated z:

Rsoln=Si30Si28soln=Xmz30CmzSimmz+XSiO230CSiO2SimSiO2-Xz30CzSimzXmz28CmzSimmz+XSiO228CSiO2SimSiO2-Xz28CzSimz	(7)

Since the solubility of zircon in the quartz-saturated solution is low, we can assume that the mass of metamict zircon dissolved equals the mass of crystalline zircon precipitated (mmz = mz). Finally, for the homogeneous model, the isotopic compositions of the precipitated zircon and the solution should be the same within error and Si isotope fractionation is small at high temperatures, so we can substitute Rz for Rsoln in Eq. (7):

mmz=CSiO2SimSiO2XSiO230-RzXSiO228CmzSiRzXmz28-Xmz30	(8)

However, zircon is continuously precipitated as the fluid changes its isotopic composition, and the precipitated zircon is unlikely to reequilibrate with the fluid by back-reaction, so the average isotopic composition of the precipitated zircon is likely significantly different from the isotopic composition of the final fluid (heterogeneous model). We will have to try some sensitivity analysis to see if assuming the homogeneous model causes significant error in the estimate of the mass of metamict zircon replaced, or do forward modeling in GWB.

As an example, take experiment Zir-016. The number of molecules of 29SiO2 dissolved in the solution is:

mSiO2≈mSi29O2molecules Si29O2=2.073×10-3g Si29O2×1 mole Si29O2 61 g Si29O2×6.022×1023molecules Si29O2mole Si29O2	(10)

Other parameters in Eq. (9) are

CSiO2Si=CmzSi=1, XSiO229=0.99,  XSiO229=0.0092, Xmz28=0.972, Xmz29=0.0467

If Rz = (Si29/Si28)z in the precipitated zircon which we assume was equal to the fluid then

δSi29=Rz-RstdRstd*1000=17,891	(11)

For the standard NBS28 Rstd = 0.05074. Plugging into Eq. (11) we obtain Rz = 0.86. Plugging values into Eq. (9) we obtain mmz = 2.7x1019 molecules ZrSiO4 = 8.2 mg ZrSiO4, which is 30.2% of the 27.17 mg of metamict ZrSiO4 added to the capsule.